N913KF – Kitfox Series 6

Before Starting Engine

- Exterior Preflight – COMPLETE

- Seat Belts – ADJUST and LOCK

- Fuel Shutoff Valve – ON

- Cowl Flaps – OPEN

- Radios, Electrical Equipment – OFF

- Rudder Pedals – ADJUST

- Brakes – TEST and SET

- Fuses – CHECK

Starting Engine

- Master Switch – ON

- Brakes – ON

- Throttle – IDLE

- Choke – AS NEEDED

- Propeller Area – CLEAR

- Ignition Switch – START

- Oil Pressure – MONITOR

- Alternator Fail Lamp – OFF

- Strobe Lights – ON

- Radios – ON

- Transponder – STANDBY

Before Takeoff

- Controls – FREE  and CORRECT

- Rudder Pedals – ADJUST

- Seat Belts – CHECK

- Cargo – SECURE

- Cabin Doors – CLOSED and LATCHED

- Flight Instruments – SET

- Fuel Quantity – ADEQUATE FOR FLIGHT

- Fuel Shutoff Valve – ON

- Runup

  - Engine - 3800 RPM

  - Ignition – CHECK

  - Engine Instruments – CHECK

  - Engine – IDLE

- Flaps – AS REQUIRED (1 notch max)

- Cowl Flaps – OPEN

- Pitch Trim – TAKEOFF

- Engine Oil Temp – MINIMUM FOR TAKEOFF

- Radios – SET

- Transponder – ON

Normal Takeoff

- Wing Flaps – SET (1 notch max)

- Throttle – FULL OPEN

- Elevator Control – FORWARD PRESSURE

- Climb Speed – 65-70 MPH (Flaps Up)

- Throttle – MAX 5500 CONTINUOUS RPM

- Trim – AS REQUIRED

Max Performance Takeoff

- Wing Flaps – 1 NOTCH

- Brakes – HOLD

- Throttle – FULL OPEN

- Brakes – RELEASE

- Elevator Control – CENTERED NEUTRAL POSITION

- Climb Speed – 55-60 MPH

- Wing Flaps – RETRACT

- Throttle – MAX 5500 CONTINUOUS RPM

- Trim – AS REQUIRED

Normal Landing

- Airspeed – 60-65 MPH

- Cowl Flaps –  AS REQUIRED

- Wing Flaps – AS REQUIRED

- Touchdown – 3 POINT

- Braking – MINIMUM REQUIRED

Go Around

- Throttle – FULL OPEN

- Airspeed – 60 MPH

- Wing Flaps – RETRACT

- Trim – AS REQUIRED

- Cowl Flaps – OPEN

Shut Down

- Lights – OFF

- Radios – OFF

- Transponder – OFF

- Ignition Switch – OFF

- Master Switch – OFF

Airspeeds

- Stall w/ Flaps	39	Vso

- Stall Clean		45	Vs1

- Climb-Angle	54	Vx

- Climb-Rate		65	Vy

- Max Flap Extend	80	Vfe

- Maneuvering	92	Va

- Normal Ops	120	Vno

- Never Exceed	140	Vne

Weights

- Max Weight	1550

- Empty Weight	790

- Useful Load	760

- Fuel 27 gals		162

- Payload w/ Fuel	598

6000’ MSL

| RPM | Fuel Gals/hour | Hours - Fuel Exhausted | Hours - VFR Reserve | GPS Speed S/MPH | TAS (Knots) | Range VFR reserve (NM) |
| --- | --- | --- | --- | --- | --- | --- |
| 4500 | 4.2 | 6.4 | 5.9 | 92 | 79 | 470 |
| 4800 | 4.8 | 5.7 | 5.2 | 101 | 88 | 456 |
| 5000 | 5.3 | 5.1 | 4.6 | 106 | 92 | 427 |
| 5100 | 5.5 | 4.9 | 4.4 | 109 | 95 | 414 |
| 5200 | 5.9 | 4.5 | 4.0 | 111 | 96 | 391 |
| 5300 | 6.3 | 4.3 | 3.8 | 114 | 99 | 374 |
| 5400 | 6.6 | 4.1 | 3.6 | 116 | 101 | 364 |
| 5500 | 6.9 | 3.9 | 3.4 | 120 | 104 | 358 |

8000’ MSL

| RPM | Fuel Gals/hour | Hours - Fuel Exhausted | Hours - VFR Reserve | GPS Speed S/MPH | TAS (Knots) | Range VFR reserve (NM) |
| --- | --- | --- | --- | --- | --- | --- |
| 4500 | 4.2 | 6.4 | 5.9 | 90 | 78 | 459 |
| 4800 | 4.8 | 5.7 | 5.2 | 100 | 86 | 450 |
| 5000 | 5.3 | 5.1 | 4.6 | 106 | 92 | 425 |
| 5100 | 5.5 | 4.9 | 4.4 | 108 | 94 | 412 |
| 5200 | 5.9 | 4.5 | 4.0 | 111 | 97 | 392 |
| 5300 | 6.3 | 4.3 | 3.8 | 113 | 98 | 369 |
| 5400 | 6.6 | 4.1 | 3.6 | 116 | 100 | 361 |
| 5500 | 6.9 | 3.9 | 3.4 | 120 | 104 | 357 |

10000’ MSL

| RPM | Fuel Gals/hour | Hours - Fuel Exhausted | Hours - VFR Reserve | GPS Speed S/MPH | TAS (Knots) | Range VFR reserve (NM) |
| --- | --- | --- | --- | --- | --- | --- |
| 4500 | 4.2 | 6.4 | 5.9 | 88 | 76 | 450 |
| 4800 | 4.8 | 5.7 | 5.2 | 101 | 88 | 457 |
| 5000 | 5.3 | 5.1 | 4.6 | 106 | 92 | 425 |
| 5100 | 5.5 | 4.9 | 4.4 | 109 | 94 | 413 |
| 5200 | 5.9 | 4.5 | 4.0 | 109 | 95 | 385 |
| 5300 | 6.3 | 4.3 | 3.8 | 111 | 96 | 363 |
| 5400 | 6.6 | 4.1 | 3.6 | 114 | 99 | 357 |
| 5500 | 6.9 | 3.9 | 3.4 | 117 | 101 | 348 |

12000’ MSL

| RPM | Fuel Gals/hour | Hours - Fuel Exhausted | Hours - VFR Reserve | GPS Speed S/MPH | TAS (Knots) | Range VFR reserve (NM) |
| --- | --- | --- | --- | --- | --- | --- |
| 4500 | 4.2 | 6.4 | 5.9 | 83 | 72 | 424 |
| 4800 | 4.8 | 5.7 | 5.2 | 97 | 84 | 436 |
| 5000 | 5.3 | 5.1 | 4.6 | 100 | 86 | 400 |
| 5100 | 5.5 | 4.9 | 4.4 | 102 | 88 | 386 |
| 5200 | 5.9 | 4.5 | 4.0 | 105 | 91 | 368 |
| 5300 | 6.3 | 4.3 | 3.8 | 108 | 93 | 352 |
| 5400 | 6.6 | 4.1 | 3.6 | 112 | 97 | 348 |